1.Memory Transfer Program

ORG 0000H

MOV R0,#30H

MOV R1,#40H

MOV R7,#05H

BACK:MOV A,@R0

MOV @R1,A

INC R0

INC R1

DJNZ R7,BACK

END

2.Parallel port interfacing of LED’S

#include <reg.51h>

void delay (unsigned int X){

int i;

for (i=0;i<X;i++)

}

void main(){

while(1){

P1=~P1;

delay(1000);

}

}

3.Square wave generation using timer with interrupt

#include <reg51.h>

sbit square_wave_pin = P1^0;

void delay(){

int i, j;

for(i=0;i<100;i++)

for(j=0;j<1275;j++);

}

void main(){

while(1){

square_wave_pin=1;

delay();

square_wave_pin=0;

delay();

}

}

Experiment 1:

8bit addition –

ORG 0000H

MOV A, #03H

MOV B, #03H

ADD A, B

END

16 bit addition

;First 16-bit number: 3456H

;Second 16-bit number: 4567H

ORG 0000H

;LSB Addition

MOV A, #56H

MOV B, #67H

ADD A, B

MOV R0, A

;MSB Addition

MOV A, #34H

MOV B, #45H

ADDC A,B

MOV R1, A

END

Experiment 2.

8bit Subtraction-

ORG 0000H

MOV A, #05H

MOV B, #03H

SUBB A, B

END

16 bit substraction

ORG 0000H

CLR C

; LSB Subtraction

MOV A, #65H

MOV B, #12H

SUBB A,B

MOV R6 ,A

;MSB Subtraction

MOV A, #57H

MOV B, #23H

SUBB A,B

MOV R3,A

END

Multiplication of 8 bit:

ORG 0000H

MOV A, #02H

MOV B, #02H

MUL AB

END

Division of 8 bit:

ORG 0000H

MOV A, #08H

MOV B, #02H

DIV AB

END

Experiment 3. Parallel Port LED Interfacing (ALL LEDS):

#include <reg51.h>

//delay function declaration

void delay(void);

void main(void)

{

//an infinite loop

while(1)

{

// Turn ON all LED's connected to Port1

P1 = 0xFF;

delay();

// Turn OFF all LED's connected to Port1

P1 = 0x00;

delay();

}

}

//delay function definition

void delay(void)

{

int i,j;

for(i=0;i<0xff;i++)

for(j=0;j<0xff;j++);

}

Experiment 4. Parallel Port LED Interfacing (ALternate LEDS):

#include <reg51.h>

void DELAY_ms(unsigned int ms_Count)

{

unsigned int i, j;

for (i = 0; i < ms_Count; i++)

{

for (j = 0; j < 100; j++); // Note: The inner loop bound is unclear, assumed to be a small value for demonstration

}

}

int main()

{

while (1)

{

P2 = 0xAA;     // Set P2 to 1010 1010b

DELAY_ms(500); // Delay 500ms

P2 = 0x55;     // Set P2 to 0101 0101b

DELAY_ms(500); // Delay 500ms

}

return (0);

}

Experiment 5. Seven Segment Display

#include <reg51.h>

// Define the segment values for numbers 0-9

unsigned char seg_code[10] = {

0x3F, // 0

0x06, // 1

0x5B, // 2

0x4F, // 3

0x66, // 4

0x6D, // 5

0x7D, // 6

0x07, // 7

0x7F, // 8

0x6F  // 9

};

void delay(unsigned int ms) {

while (ms--) {

unsigned int i;

for (i = 0; i < 123; i++); // Roughly 1 ms delay

}

}

void display(unsigned char num) {

P1 = seg_code[num]; // Send the segment code to port P1

}

void main() {

unsigned char i;

while (1) {

for (i = 0; i < 10; i++) {

display(i); // Display numbers 0-9

delay(1000); // Delay for 1 second

}

}

}

Experiment 6.Switch ,Relay and Buzzer

#include <reg51.h>

sbit Buzzer = P3^7;

void  delay_ms(unsigned int time)	   //This Function is used to cause delay between LED ON and OFF events

{

unsigned int  i, j;

for (j=0; j<time; j++)

for(i=0; i<113; i++);

}

void Ext_Int1 (void) interrupt 2		//0013h

{

Buzzer = ~Buzzer;

delay_ms(250);

}

void main() 			//the program execution stat point//

{

Buzzer = 1;

INT1 = 1;

EX1 = 1;

EA = 1;

while(1);

}

LCD DISPLAY:

#include <reg51.h>

//Declarations

#define LCD_DATA    P2				//LCD data port to PORTD

sbit rs = P0^0;

sbit rw = P0^1;

sbit en = P0^2;

//Function Prototypes

void init_LCD(void);					//Function to initialize the LCD

void LCD_command(unsigned char cmd);	//Function to pass command to the LCD

void LCD_data(unsigned char  val);		//Function to write character to the LCD

void LCD_write_string(char *str);//Function to write string to the LCD

void msdelay (unsigned int time);		//Function to generate delay

//Start of Main Program

void main(void)

{

char var1[] = "LCD Interface";//Declare message to be displayed

char var2[] = "Test Program";

init_LCD();           	// call function to initialise of LCD

msdelay(50);       	// delay of 50 mili seconds

LCD_write_string(var1);//Display message on first line

msdelay(15);

LCD_command(0xC0);      	// initiate cursor to second line

LCD_write_string(var2);//Display message on second line

while (1);				//Loop here

}							//End of Main

//Function Definitions

void msdelay (unsigned int time) //Function to generate delay

{

unsigned int i, j;

for (i = 0; i < time; i++)

for (j = 0; j < 113; j++);//Calibrated for a 1 ms delay in MPLAB

}

void init_LCD(void)		// Function to initialise the LCD

{

LCD_command(0x38);      // initialization of 16X2 LCD in 8bit mode

msdelay(15);

LCD_command(0x01);      // clear LCD

msdelay(15);

LCD_command(0x0C);      // cursor off

msdelay(15);

LCD_command(0x80);      // go to first line and 0th position

msdelay(15);

}

void LCD_command(unsigned char cmd) //Function to pass command to the LCD

{

LCD_DATA = cmd;		//Send data on LCD data bus

rs = 0;				//RS = 0 since command to LCD

rw = 0;				//RW = 0 since writing to LCD

en = 1;				//Generate High to low pulse on EN

msdelay(15);

en = 0;

}

void LCD_data(unsigned char val)//Function to write data to the LCD

{

LCD_DATA = val;	//Send data on LCD data bus

rs = 1;				//RS = 1 since data to LCD

rw = 0;				//RW = 0 since writing to LCD

en = 1;				//Generate High to low pulse on EN

msdelay(15);

en = 0;

}

//Function to write string to LCD

void LCD_write_string( char *str)

{

int i = 0;

while (str[i] != 0)

{

LCD_data(str[i]);      // sending data on LCD byte by byte

msdelay(15);

i++;

}

}