1. Study of Generation & Reception of BPSK. kit

2 Study of Generation & Reception of BFSK kit

3 : Study of Generation & Detection of baseband signal using Matched Filter matlab code kit not available

Code :

clc; clear;

% Parameters

N = 10;                         % Number of bits

fs = 100;                       % Samples per bit

bits = randi([0 1], 1, N);      % Random bits

nrz = 2*bits - 1;               % NRZ: 0→-1, 1→+1

% Pulse shaping (rectangular pulse)

pulse = ones(1, fs);

tx = kron(nrz, pulse);          % Transmitted signal

% Add noise

snr = 10;                       % SNR in dB

P = mean(tx.^2);                % Signal power

noise = sqrt(P/10^(snr/10)) * randn(1, length(tx));

rx = tx + noise;               % Received signal

% Matched filter (same as pulse shape)

mf = fliplr(pulse);

y = conv(rx, mf, 'same');

% Sample at bit centers

samples = y(fs/2:fs:end);

detected = samples > 0;

% Display results

disp('Transmitted bits:');

disp(bits);

disp('Detected bits:');

disp(detected);

% BER

ber = sum(bits ~= detected)/N;

disp(['BER: ', num2str(ber)]);

OUTPUT:

4. Study of Generation & Reception of DSSS. KIT

5 Simulation of Random Process. Matlab code

CODE:

A = 1;                      % Amplitude

f = 5;                      % Frequency in Hz

T = 1;                      % Total time duration in seconds

fs = 1000;                  % Sampling frequency

t = 0:1/fs:T;               % Time vector

N = 10;                     % Number of random process realizations

% Initialize matrix to hold realizations

X = zeros(N, length(t));

% Generate realizations

figure;

hold on;

for i = 1:N

phi = 2*pi*rand();      % Random phase in [0, 2*pi]

X(i, :) = A * cos(2*pi*f*t + phi);

plot(t, X(i, :));       % Plot each realization

end

title('Random Process Realizations: A cos(2πft + φ)');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

hold off;% Parameters

OUTPUT:

6. Simulation of performance of Binary PSK receiver in presence of noise MATLAB CODE

clc;

clear;

% Parameters

N = 1e6;                          % Number of bits

EbN0_dB = -4:1:10;                % Eb/N0 range in dB

ber_sim = zeros(size(EbN0_dB));   % Preallocate for speed

% Generate random bits

bits = randi([0 1], 1, N);

% BPSK Modulation: 0 -> -1, 1 -> +1

symbols = 2*bits - 1;

for i = 1:length(EbN0_dB)

% Convert Eb/N0 from dB to linear

EbN0 = 10^(EbN0_dB(i)/10);

% Noise standard deviation (since Es = 1 for BPSK)

noise_std = sqrt(1 / (2 * EbN0));

% Generate AWGN

noise = noise_std * randn(1, N);

% Transmit over AWGN channel

received = symbols + noise;

% Receiver: Hard decision decoding

detected_bits = received > 0;

% Count bit errors

ber_sim(i) = sum(bits ~= detected_bits) / N;

end

% Theoretical BER for BPSK in AWGN

EbN0_lin = 10.^(EbN0_dB / 10);

ber_theory = qfunc(sqrt(2 * EbN0_lin));

% Plotting

figure;

semilogy(EbN0_dB, ber_sim, 'bo-', 'LineWidth', 2);

hold on;

semilogy(EbN0_dB, ber_theory, 'r--', 'LineWidth', 2);

grid on;

xlabel('E_b/N_0 (dB)');

ylabel('Bit Error Rate (BER)');

title('BER Performance of BPSK over AWGN Channel');

legend('Simulated BER', 'Theoretical BER');

axis([-4 10 1e-6 1]);

OUTPUT:

7. Simulation study of various Entropies and mutual information in a

communication system MATLAB CODE

>> clc;

clear;

% Define Channel Transition Matrix P(Y|X)

% Example: Simple 2x2 Channel

P_Y_given_X = [0.8 0.2;   % P(Y=1|X=1), P(Y=2|X=1)

0.1 0.9];  % P(Y=1|X=2), P(Y=2|X=2)

% Define Input Distribution P(X)

P_X = [0.6, 0.4];  % Probability distribution for X = [X1, X2]

% Number of input and output symbols

[num_outputs, num_inputs] = size(P_Y_given_X);

% Compute Joint Distribution P(X, Y)

P_XY = zeros(num_outputs, num_inputs);

for x = 1:num_inputs

for y = 1:num_outputs

P_XY(y,x) = P_X(x) * P_Y_given_X(y,x);  % Joint probability P(X, Y)

end

end

% Compute Marginal Distribution of Y

P_Y = sum(P_XY, 2);  % P(Y)

% Compute Entropies

% H(X) - Entropy of input distribution

H_X = -sum(P_X .* log2(P_X + eps));  % Add eps to avoid log2(0)

% H(Y) - Entropy of output distribution

H_Y = -sum(P_Y .* log2(P_Y + eps));  % Add eps to avoid log2(0)

% H(Y|X) - Conditional entropy of Y given X

H_Y_given_X = 0;

for x = 1:num_inputs

for y = 1:num_outputs

p_y_given_x = P_Y_given_X(y, x);

if p_y_given_x > 0

H_Y_given_X = H_Y_given_X - P_X(x) * p_y_given_x * log2(p_y_given_x);

end

end

end

% Mutual Information I(X;Y)

I_XY = H_X - H_Y_given_X;

% Display Results

fprintf('H(X)        = %.4f bits\n', H_X);

fprintf('H(Y)        = %.4f bits\n', H_Y);

fprintf('H(Y|X)      = %.4f bits\n', H_Y_given_X);

fprintf('I(X;Y)      = %.4f bits\n', I_XY);

OUTPUT: